# Design Review

# Sweeper Drive Concept

Date:  Thursday, March 2, 2017

Location:  WSMSA

Attendees

Geo, Jen, Mentor Jay, Mentor George

| Major Milestones: |  | Completion Goal | Completed |
| --- | --- | --- | --- |
| Worlds Competition – might go. |  | Apr 2017 |  |
| Post competition – Hatter’s Scrimmage |  | May 16, 2017 |  |

Meeting Notes

Agenda:

- Review initial CAD, sweeper redesign

Current issues:

- The feeder does not aggressively pick up particles, and jams frequently.

- Finger mounting to the rollers is ugly (but easy) (pierce tube with sheet metal screw - ick).

- Picture of the current arrangement – see figure 1 attached.

Observations:

- The sweeper fingers seem to be too flexible – they bend really easily, but may not be causing the jamming.

- The drive belts/rings slip easily on the rollers.

- Good tension to the motor pulley is not achieved, even with the idler pulleys.

- When picking up particle, if there is any resistance, the belts slip, the particle stops moving up the ramp.

- The motor keeps running, the drive belts slip on the motor pulley.

- The drive motor mount is hitting one of the drive wheel motors, preventing the particle ramp from being mounted in the correct place.

- Motor to roller spacing is not correct for the belts we have.

- Ramp to roller spacing is incorrect, perhaps too small to let the particles pass the roller when the sweeper fingers contact the particle.

Suggested concepts for improvement:

- See the attached figure 3.

- Move the sweeper drive motor

- Change from O-Ring belt drive system to geared system, gearing the rollers to move faster than the motor.

- Re-attach the fingers

Review comments:

- Looks feasible

- Gears used by other teams, with good results.

- Jay – need a guard to keep the fingers from going into the gears.

- Better if the motor didn’t stick out of the side panel

- How to hold the fingers – put plastic balls inside the tubing.  Tubing goes through the roller, one ball from each end.

- Use stiffer tubing, but instead of 4 fingers at each spot go with 2.

- Bottom roller gear sticks outside robot allowed size – too big.

| Action Items | Assigned | Date/Goal |
| --- | --- | --- |
| Find smaller gears (to drive rollers) | N/A | 3/05 - Ordered |
| Find stiffer tubing | Chandler | 3/10 |
| Rework rollers for stiffer tubing, reassemble | Chandler | 3/20 |
| Figure out size and purchase small plastic balls. | Chandler | 3/20 |
| Revise design to include finger guards at gears | Engineering | 3/20 |
| Concept different motor mounting so the motor doesn’t stick out. | Engineering | 3/20 |

Status as of 3.05.2017:

- Smaller gears have been ordered and shipped.  Yet to be received.

- Redesign with guard and different motor mounting pending.

| Prepared By: | Chandler | Date: |  |
| --- | --- | --- | --- |
| Overview By: | G. Edward | Date: |  |